There are 3 Fundamental Steps in the production of any Lithographic Printing Plate.

- Image Formation 2) Development 3) Desensitizing

What is meant by Desensitization? the non-image areas of a litho plate must be treated so they become water receptive. Good Desensitizing Agents are responsible for the adsorption of these agents onto the Aluminium surface inducing desensitizing properties.

Step 3 is omitted by DoP plates, which mean – high surface water film level thickness dampening to keep the plate clean. You become reliant on how efficient is your F.S

On desensitized treated plates, a press wide sheet of water film thickness measures 0.3 Microns. IMHO using DoP and Polyester Plates requires high dampening levels of 0.5 – 0.6 Microns.

Regards, Alois